[image: image1.png]Media Converter 1

Twist Pair

TX| [RX  Rus

OPTICAL FIBER

=)
ETHERNET 1 ETHERNET 2

TX RX RS
TwistPair ___Media Converter2






Fiber Media converter


  The non-managed media converters, which give you economic solutions for media conversion between fiber and copper Ethernet. It provides you SC,FC or ST connectors for your fiber optic cables and RJ45 for twisted pair UTP cable connection.The maximum extension is up to 100km over fiber. 
Features                                                         
* In conformity to IEEE802.3 10base-T
* Hot Plug-gable & Wall-Mountable.
* Stable performance more than 50thousand hours with fault-free.        

* Unique IC solution applied.
* Supports 10Mbps full/half duplex, easy upgrades.
* Auto MDI/MDI-X.

* Auto Negotiation.

* Support flow control.
* Concentrated into EF16-D220(16slot for media converter cards) or EF14-D220(14slot for media converter standalone type) 2U rack-mount chassis.
* In conformity with safety code of FCC, CE and RoHS.

Specifications

	Optical Interface
	Connector
	1x9 SC/FC/ST

	
	Data Rate
	10Mbps

	
	Duplex Mode
	Full duplex

	
	Fiber
	MM 50/125um,62.5/125um

SM 9/125um

	
	Distance
	10/100Mbps:

MM 2km,SM 25/40/60/80/100/120km

10/100/1000Mbps:

MM 550m/2km,SM 20/40/60/80/100km

	
	Wavelength
	MM 850nm,1310nm

SM 1310nm,1550nm

WDM Tx1310/Rx1550nm(A side),Tx1550/Rx1310nm(B side)

WDM Tx1490/Rx1550nm(A side),Tx1550/Rx1490nm(B side)

	UTP Interface
	Connector
	RJ45

	
	Data Rate
	10Mbps

	
	Duplex Mode
	Half/full duplex

	
	Cable
	Cat5,Cat6

	Power Input
	Adapter Type
	DC5V

	
	Power Built-in Type
	AC100~240V

	Power Consumption
	＜3W

	Weight
	Adapter Type
	0.39kg

	
	Power Built-in Type
	0.67kg

	Dimensions
	Adapter Type
	94*70.5*26.5cm

	
	Power Built-in Type
	140.5*111*30cm

	Temperature
	0~50℃ Operating; -40~70℃ Storage

	Humidity
	5~95%(no condensing)

	MTBF
	≥10.0000h

	Certification
	CE,FCC,RoHS


Application
* Extend your Ethernet connection up to 0~100km away using fiber optics

* Creates an economical Ethernet-fiber/copper-fiber link for connecting remote sub-networks to larger fiber optic networks/backbones

* Converts Ethernet to fiber, fiber to copper/Ethernet, ensuring optimum network scalability for connecting two or more Ethernet network nodes (e.g. connecting two buildings on the same campus)

* Designed to provide high-speed bandwidth for demanding large scale work groups that require expansion of Gigabit Ethernet Network

[image: image6.jpg]e ]|





System Design

1. For Networking

[image: image2.png]Switch

Media Converter

Fiber

2URack

Laptop.




For CCTV Surveillance Security

[image: image3.png]1P Camera

v




Product Pictures

	10M Dual Fiber

[image: image4.jpg]



	10M Single Fiber

[image: image5.jpg]




	
	


Order Information

	Part Number
	Rate
	Optic fiber
	Wavelength

(nm)
	Power

(dBm)
	Rx sensitivity

(dBm)
	Reach

(KM)
	Loss allowed

(dBm)

	850MM-2
	10Mbps
	MM
	1310
	-20~-12
	-30
	2
	10

	850SM-25
	10Mbps
	SM
	1310
	-15~-7
	-34
	25
	19

	850SM-40
	10Mbps
	SM
	1310
	-9~-5
	-36
	40
	27

	850SM-60
	10Mbps
	SM
	1310
	-5~-2
	-36
	60
	31

	850SM-80
	10Mbps
	SM
	1550
	-5~0
	-37
	80
	32

	850SM-100
	10Mbps
	SM
	1550
	-3~0
	-38
	100
	35

	850SM-2A/B
	10Mbps
	MM
	1310/1550
	-14~-8
	-30
	2
	17

	850SM-25A/B
	10Mbps
	SM
	1310/1550
	-8~-3
	-33
	25
	25

	850SM-40A/B
	10Mbps
	SM
	1310/1550
	-8~-3
	-33
	40
	25

	850SM-60A/B
	10Mbps
	SM
	1310/1550
	-5~0
	-37
	60
	30

	850SM-80A/B
	10Mbps
	SM
	1490/1550
	-0~0
	-39
	80
	34

	850SM-100A/B
	10Mbps
	SM
	1490/1550
	-0~0
	-39
	100
	34


